For both scenarios, the previous creation of two virtual environments is needed (“yolo_termica” and “yolo_tracking”) along with the dependencies and modules for each camera setup.

The scripts that perform detection (real-time or with an input video) generate .csv files (all of them with the exact same columns/fields) in order to be compared later on the final step of our system. This is done by executing the program compare_csv.py. With this, we obtain a truthful and complete analysis of the comparison of both camera feeds, alerting of possible incidents

.

REAL-TIME DETECTION

For the thermal camera

- Activate virtual environment for the thermal camera

source yolo_termica/bin/activate

- Go to the directory where the dependencies and modules needed are stored

cd yolo_termica/pysenxor-master

- Execute the script for real-time detection with python3 thermal_detection.py

For the csi camera with tracking

- Activate virtual environment for the csi camera

source yolo_tracking/bin/activate

- Go to the directory where the dependencies and modules needed are stored

cd tracking/machine-learning/yolo_tracking

- Execute the script for real-time detection with python3 tracking.py

RECORD AND PERFORM DETECTION AFTER

For the thermal camera (Steps 1 and 2 are the same as before)

- Execute the script for recording with python3 thermal_recording.py

(Here we need to introduce a name for the video that will be recorded and the maximum duration of the recording, the video will be saved at the folder “experiments”)

- Once the video is recorded, execute the code for the detection

python3 thermal_detection_from_video.py

For the csi camera with tracking (Steps 1 and 2 are the same as before)

- Execute the script for recording with python3 csi_recording.py

(Here we need to introduce a name for the video that will be recorded and the maximum duration of the recording, the video will be saved at the folder “experiments”)

- Once the video is recorded, execute the code for the detection

python3 tracking_from_video.py